## WHEEL ALIGNMENT

| NOTE: | Specifications are in degrees. |
| --- | --- |

### FRONT

| ### FRONT WHEEL ALIGNMENT | ### PREFERRED SETTING | ### ACCEPTABLE RANGE |
| --- | --- | --- |
| CAMBER - RT/LT | -0.375° | –0.750° to 0° |
| MAX RT/LT DIFFERENCE | 0.7° | 0.7° |
| CASTER -- RT/LT | 3.3° | 2.8° to 3.8° |
| MAX RT/LT DIFFERENCE | 0.5° | 0.5° |
| TOE - TOTAL* | 0.2° | +0.075° to +0.325° |
| Positive toe (+) is toe-in and negative toe (−) is Toe-out. |  |  |
| * TOTAL TOE is the sum of both the left and right wheel toe settings. TOTAL TOE should be equally split between each wheel on the same axle to ensure the steering wheel is centered after setting toe. |  |  |

### REAR

| ### REAR WHEEL ALIGNMENT | ### PREFERRED SETTING | ### ACCEPTABLE RANGE |
| --- | --- | --- |
| CAMBER | –0.25° | –0.625° to +0.125° |
| TOE - TOTAL* | 0.25° | –0.16° to +0.66° |
| THRUST ANGLE | 0.00° | –0.25° to +0.25° |
| Positive toe (+) is toe-in and negative toe (−) is Toe-out. |  |  |
| * TOTAL TOE is the sum of both the left and right wheel toe settings. TOTAL TOE should be equally split between each wheel on the same axle to ensure the steering wheel is centered after setting toe. |  |  |